EXERCISE ON SAMPLE SIZE DETERMINATION

We will use the example from the Nepal NTP, Far-Western Region, shown below.

Study the compiled reports for 2000, with annual total volume, positive, negative and scanty smears examined.

Question 1

Is the information given sufficient to determine sample sizes? Please make the necessary calculations, and explain why you need to do this.

Question 2

What else do you need to know to look up required sample sizes in the LQAS tables?

NTP NEPAL, AFB Laboratories performance data

| District |  | Centre |  | Annual Volume |  |  | Negative Slides | Positive Slides |  |  | Scanty Slides |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ACH |  | DHO |  | 627 | 537 |  |  | 86 | 4 |  |  |
| 2 | BAI |  | DHO |  | 888 | 846 |  |  | 42 | 0 |  |  |
| 3 | BAI |  | PATAN |  | 90 | 78 |  |  | 10 | 2 |  |  |
| 4 | BAJH |  | DHO |  | 372 | 291 |  |  | 73 | 8 |  |  |
| 5 | BAJU |  | DHO |  | 144 | 126 |  |  | 15 | 3 |  |  |
| 6 | DAND |  | DHO |  | 120 | 120 |  |  | 0 | 0 |  |  |
| 7 | DAND |  | T HOSP |  | 2229 | 2100 |  |  | 112 | 17 |  |  |
| 8 | DARCH |  | DHO |  | 1173 | 1053 |  |  | 110 | 10 |  |  |
| 9 | DARCH |  | LALI |  | 114 | 114 |  |  | 0 | 0 |  |  |
| 10 | DOIT |  | KEDAR |  | 45 | 45 |  |  | 0 | 0 |  |  |
| 11 | DOTI |  | D HOS |  | 843 | 738 |  |  | 93 | 12 |  |  |
| 12 | KAIL |  | BAHJ |  | 609 | 495 |  |  | 107 | 7 |  |  |
| 1 | KAIL |  | CHAU |  | 258 | 240 |  |  | 18 | 0 |  |  |
| 14 | KAIL |  | DODO |  | 1071 | 990 |  |  | 78 | 3 |  |  |
| 15 | KAIL |  | SETI |  | 5466 | 4679 |  |  | 725 | 62 |  |  |

Question 3

Now also study the plot in this figure.

What is your impression considering the variation in key parameters needed to set sample size for rechecking? Is it possible to use a uniform sample size in this situation? Please explain.

Question 4

The first screening as well as second control on discordants will have to be done in the Regional Hospital. State the advantages and disadvantages of making the regional hospital responsible for the rechecking of the slides. The Head Technician has enough spare time to check the discordants, but the only available AFB-technician already has 2500 smears to read annually.

Question 5

Estimate how many additional rechecking slides this technician could screen per year

Question 6

You are proud of your service, and you think they should reach at least 85% sensitivity, and could have a maximum of one false negative error. Use the table below to look up individual and standard-average sample sizes for the parameters specified. The LQAS tables needed can be found in the EQA Guide, appendix D4.

| District |  | Centre | Annual Volume | Negative Slides | Positive Slides | Scanty Slides | % Pos | Sens: 85% d=1 | Sens: .. d= .. (Question 10) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ACH | DHO | 627 | 537 | 86 | 4 | 13.7% |  |  |
| 2 | BAI | DHO | 888 | 846 | 42 | 0 | 4.7% |  |  |
| 3 | BAI | PATAN | 90 | 78 | 10 | 2 | 11.1% |  |  |
| 4 | BAJH | DHO | 372 | 291 | 73 | 8 | 19.6% |  |  |
| 5 | BAJU | DHO | 144 | 126 | 15 | 3 | 10.4% |  |  |
| 6 | DAND | DHO | 120 | 120 | 0 | 0 | 0.0% |  |  |
| 7 | DAND | T HOSP | 2229 | 2100 | 112 | 17 | 5.0% |  |  |
| 8 | DARCH | DHO | 1173 | 1053 | 110 | 10 | 9.4% |  |  |
| 9 | DARCH | LALI | 114 | 114 | 0 | 0 | 0.0% |  |  |
| 10 | DOIT | KEDAR | 45 | 45 | 0 | 0 | 0.0% |  |  |
| 11 | DOTI | D HOS | 843 | 738 | 93 | 12 | 11.0% |  |  |
| 12 | KAIL | BAHJ | 609 | 495 | 107 | 7 | 17.6% |  |  |
| 1 | KAIL | CHAU | 258 | 240 | 18 | 0 | 7.0% |  |  |
| 14 | KAIL | DODO | 1071 | 990 | 78 | 3 | 7.3% |  |  |
| 15 | KAIL | SETI | 5466 | 4679 | 725 | 62 | 13.3% |  |  |
| Total |  |  | 14049 | 12452 | 1469 | 128 | 10.5% |  |  |

Question 7

What is the total number of slides that will need to be rechecked annually by this regional technician?

Question 8

Is this a feasible workload, allowing him to work accurately so as to arrive at results that are more reliable than those of his centres?

Question 9

If it would seem that this workload is excessive, what can you do to make technically accurate rechecking feasible, apart from adding staff? Please work out your solution in the table above.

Question 10

Calculate total workload for these two sets of parameters, but using a uniform sample size for all centres, based on average positivity prevalence and negatives volume.

What does it mean for total workload compared to individual sample sizes?

Question 11

What are the other disadvantages of such an approach? Of all these systems, which would be most appropriate and efficient in this setting, and why?