EXCEL WORKBOOK “EVALUATION OF AFB-MICROSCOPY LABORATORIES” FOR COMPUTERIZED COMPILATION AND ANALYSIS OF SMEAR MICROSCOPY LABORATORY PERFORMANCE

| Purpose: | To give participants an insight in the function and utility of the workbook tool, and teach them how to use it effectively |
| --- | --- |
| Learning objectives: | At the end of the module, the participant will be able to: - Describe the use and limitations of the workbook - Name the different parts of the workbook and their purpose - Describe the steps for setting up the workbook - Perform data entry - Understand the data analysis performed by the computer - Prepare reports and listings of problem laboratories |
| Content Outline: | - Description of the workbook format and uses - Details of workbook parts and sheets - How to use the workbook set-up sheet - How to perform data entry - How to prepare reports and listings - How to prepare a workbook ready for the coming year |
| Handout and exercises: | None |
| Appendix: | None |

Description of the workbook format and uses

There are two versions of this workbook. One is for use at the national level, containing sufficient lines to enter data for a maximum of 1500 laboratories; it can produce analyses by up to three different administrative levels, re-grouping their laboratories. The highest administrative division (for instance the zones or divisions) can have up to 50, the second one (for instance the regions) up to 100, and the third (e.g. districts) up to 500 units.

The second, smaller version can be used at national level in smaller countries or at intermediate administrative level where appropriate. It works in the same way, but can accommodate only 250 laboratories and one administrative level sub-analysis with up to 31 different units (i.e. districts).

The workbook has a menu for easy navigation between the numerous sheets, a set-up page for the names of the administrative divisions, laboratories and controllers, sheets for quarterly data entry, annual compilation sheets, and sheets for various analyses and reports.  It will automatically make the additions quarterly and annually, by sheet, by peripheral microscopy laboratory, by one or more higher administrative division(s), and by first controller. It will also automatically analyze these results, provided that all the information needed has been entered. In particular, the performance data for each laboratory (i.e. numbers of smears examined) are needed to analyze the rechecking results objectively.

The analysis of the performance data includes calculation of total workload, prevalence of positives among diagnostic versus follow-up smears, and the total prevalence for positives or scanty positive smears. These are all indicators which should be checked when probing for problems.

The rechecking results analysis takes into account differences in prevalence of positives between laboratories, as known from the performance reports, and between the samples presented to the first controllers and the routine slides. This is done by calculating the percentage of true positive & scanty detected by the controllers (= the relative sensitivity of their work) overall and by controller; the quality of the work of each peripheral lab is compared to that of the controllers’ average by calculating a ratio which compares quality of the detection by the peripheral lab with that of the controllers.  In addition to the analyses for each laboratory, the rechecking results of each controller are analyzed, based on these numbers only. This analysis, and also those re-grouped by administrative division, will be made by the computer only after completing the data entry, then saving, closing and re-opening the workbook.

Finally there is one summary sheet, summarizing the essential information from various sheets, which can be useful as a brief annual report on smear microscopy laboratory network performance. The large workbook can be used to compile a national report, entering data manually, or by copying and pasting from smaller workbooks compiled at the intermediate level and sent to national as electronic files. All sheets can be printed, or part of their contents copied to a new sheet, for summary reporting to the higher level, or for feedback to the lower level.

All sheets except those meant for data entry are protected, but there is no password. Formatting remains possible. It is strongly recommended not to remove the protection, since this could result in erasing formulas by accident during data entry, which is prevented as long as the protection is kept on. Once proper functioning is assured, it is strongly recommended to add a password to the protection, not known to those entering the data.

The sheets are very large, because it is very difficult to add more lines without disturbing the formulas and calculations. If fewer lines are needed, the empty lines between the last lab and the totals at the bottom of each sheet should simply be hidden. To do this, select these lines in the left margin, click the right mouse button, and click “hide”. They can easily be unhidden again selecting the lines above and beneath the hidden part and clicking the right mouse button, “unhide”.

DON’Ts:

- NEVER DELETE LINES THAT ARE NOT NEEDED, which should be prevented if the protection is left on.

- NEVER ATTEMPT TO SORT SHEETS OR INSERT LINES in any sheet. Since all sheets are inter-linked, this would disturb the correct analyses profoundly.

Details on workbook parts and sheets

The workbook contains several worksheets, which can be accessed through links on the opening Menu sheet. They are also labeled at the bottom of each sheet, clicking the “tab” will open the respective sheet.  Besides the Menu sheet, these are from left to right:

- Set up: must be completed first, before data entry, since this will allow the names of laboratories etc.to be copied to all other sheets automatically. Information needed: name of the reporting administrative division (or project, or country); year; designation of the administrative divisions below the reporting unit (one to three); names of the laboratories with its division (and sub-divisions), all aligned on the same line. There is also one column to enter the names of the first controllers, these will then be available as a list during data entry.

- Q1 to Q4: these sheets will be used to enter data from the quarterly results of rechecking, for each lab and also for the first controller. The columns on the left automatically show the names of the laboratories covered in the report, and their administrative unit(s). The identification code or name of the first controllers must be entered in each sheet and on each line; this can be done by clicking on the arrow of the box and choosing the correct one from the drop-down list that appears.

- Q1P to Q4P: these sheets are used to enter data from quarterly performance (“output”) reports of the laboratories. Again, the columns on the left automatically show the names and administrative sub-units of the laboratories covered in the report.

- YR EQA: shows the totals of the quarterly rechecking data already entered. No data needs to be entered on this sheet, it will automatically add up the quarters.

- YR P: shows the totals and analysis of the routine lab performance data entered in Q1P through Q4P, as far as already entered. On the right is an analysis, showing totals and percentage of positive/scanty results for diagnostic and follow-up smears separately. No data needs to be entered on this sheet, it will automatically add up the quarters.

- YR CONTR: shows the totals of the quarterly rechecking data already entered, summarizing the portion of these sheets on the right side, which are the first controller results. Each controller is shown once, with percentage true positive detected (or relative sensitivity) and percentage true positive/all positives (predictive value) that are automatically calculated. No data needs to be entered on this sheet, it will automatically add up the quarters, but data newly added to the data sheets for a quarter will be calculated only after closing and reopening the workbook.

- Analysis_Labs: also here nothing has to be entered. The sheet shows summary analysis of routine AFB-microscopy performance, as well as the findings from rechecking, by peripheral laboratory, using the sums of all quarters entered so far.

  - To the right of the name appear the total number, percent positive and percent scanty results for “Smears examined during the year”, in this case without distinguishing “diagnostic” versus “follow-up”. The next columns show the totals of smears rechecked, by result. The right hand part shows selected results of rechecking: number of HFP, HFN and LFN, along with their analysis (“detection proportional to the controllers”; “percent true positives out of all positives”). In the top right corner appears one average estimate of controllers’ performance (“Percentage of true positive/scanty overall detected by the controllers”, which is in fact, sensitivity).

  - Some of the analysis columns have been given color codes to automatically highlight unsatisfactory performance. Different levels have a different color, which can be changed via the “Format” menu, “Conditional formatting”.

  - These columns also have autofilters as shown by the downward arrows in their heading boxes, allowing for fast searches or selections. Clicking one of these shows the list of values in that column, and other options. To search for a laboratory (or district or certain value of one of the parameters) using these tabs is easy since the names or values appear sorted alphabetically, clicking one will bring up its line. Using the filters is described in more detail in the section “How to prepare reports and listings” later in this module.

  - Analysis of rechecking results is shown for each lab as “detection proportional to the controllers”, comparing the lab to the average of the first controllers. If this value is around 1.00, the lab and controllers detects positives equally; a value clearly below 1.00 means that the lab is not as good at detecting true positives, while above 1.00 means the lab is better than the controllers. The other evaluation shown concerns the false positives, “percent true positives out of all positives”. Scanty results are not included here, since they cannot always be confirmed by rechecking. Besides this percentage, or in fact the positive predictive value, the number of HFP detected should be taken into account since the total positives rechecked may have been too low to yield a meaningful percentage. For the same reason, this percentage is only calculated and will only appear if more than 4 positives were rechecked.

- Summary_Labs: nothing needs to be entered on this sheet. The sheet automatically produces a summary of performance and quality of all laboratories, as far as they were included in the performance and rechecking reports. The number of laboratories is shown, based on the list entered in the set-up sheet, and the number rechecked (at least on one or a few smears). This sheet then shows a classification of laboratories based on quality of positive (“true positives”) and negative (“detection”) results, which is a very important summary parameter to see the level and progress of the area or country. The sheet could thus serve as a report on the TB microscopy lab services, to be included in a general NTP report. The number of laboratories with insufficient information to analyse quality as documented by rechecking (see above) also appears. Please note that the laboratories with less than 5 positives rechecked appear as insufficient information for calculation of the “percent true positives out of all positives” as well.

- Admin_Div_totals (small version workbook): this sheet shows rechecking totals for the administrative subunits, provided these were specified. In the title, the designation of the division is automatically adapted to how it is named in the set-up sheet. This sheet is automatically recalculated each time the workbook is opened. It will not be updated during data entry, but only after the workbook has been saved, closed and opened again. It contains the same information as the summary report for the first controllers.

- “Admin_Div_III_Totals”: In the larger version, the sheet “Admin_Div_totals” is now called “Admin_Div_III_Totals”, and there are two more of these sheets to make the analyses re-grouped by higher administrative division, “Admin_Div_II_Totals” and “Admin_Div_I_Totals”. They function in the same way, updates are made only after saving, closing and reopening the workbook.

- “Calculations”, “Pivot_regions” and “Pivot_contr”: these are three hidden sheets that contain intermediary calculations needed for some of the analyses. The last two cannot be protected for automatic functioning and are complicated, so they should remain hidden to reduce the risk that they are accidentally changed.  This must be done by protecting the complete workbook, and adding a password.

How to use the workbook set-up sheet

Information needs to be added to this only once for the whole book and year, unless new laboratories need to be added during the year.

Using the smaller version, from the top down:

- first enter the designation and name of the highest administrative reporting unit to which all laboratories reported on in the workbook belong. On the right, add the designation of one lower administrative division by which you want sub-analyses to be performed (for instance “Region”, or “District”). This designation now also appears in the header of the column on the left below.

- below these division boxes, enter the year of reporting.

- in the columns further down, the names of the laboratories and their corresponding administrative division (chosen level) should be entered on the same line, add each lab only once. It is more practical to do this by administrative division, so that its name needs to be typed one time, to be copied downwards over all the laboratories in this division. This will avoid duplicates appearing in the sub-analysis because of different spelling of the name entered by mistake. These names will now appear in all other sheets in the same place, where they cannot be changed or added. In fact, laboratories and their administrative divisions can be entered in any sequence. So, new laboratories to be added later on during the year should be added on the first empty line at the bottom. DO NOT ATTEMPT TO INSERT NEW LABS OR SORT THIS TABLE, it will result in corrupting the workbook entirely, and it is unnecessary for analysis.

- the right hand column has a list of codes, which should be replaced by the names of the first controllers. Their names should be entered only once, and they do not need to be aligned with any laboratory or administrative division. These will then be available as a drop-down list in the appropriate place when entering the quarterly results of rechecking (Q1 to Q4).

Country version, from the top down:

- first enter the name of the country, followed by the designations of up to three administrative divisions, for instance “Zone / Region / District”. These designations will now also appear in the header of the respective columns below.

- below these division boxes, enter the year of reporting.

- in the columns further down, the names of the laboratories and their corresponding administrative division (chosen level) should be entered on the same line, and each lab only once, as described above.

- also the list of first controllers should be entered as for the smaller workbook

How to perform data entry

Rechecking results must be entered in the sheets Q1 to Q4 for the respective quarters. Missing results or quarters should simply be left blank. The numbers to be entered must be taken from the compilation quarterly rechecking report Form 3, or directly from Form 1 for a single laboratory. Find the name of the laboratory in the columns on the left, and to their right enter the numbers of smears rechecked (by result) and the numbers or errors. The same is done for the first controllers, in the right part of the sheets and on the same line, per laboratory. Note: enter only NUMBERS in those boxes, nothing else, otherwise the calculations will not work!!

In between these two parts is a column for the name of the first controller, which should be taken from a drop-down list after clicking on the arrow that will appear in the right corner of these cells. It can also be typed, but only names or codes corresponding to the list in the set-up will be accepted.

In the header of the first quarter sheet, the names of the second controller and EQA coordinator can be entered, and will then also appear in all sheets as appropriate.

Laboratory performance data must be entered in the sheets Q1P to Q4P. In the columns to the right of the names of labs and their administrative divisions the numbers of smears routinely read should be entered, by result and by type, divided as diagnostic or follow-up. Totals are NOT entered, they will be added up in the annual sheet by the computer. Note: enter only NUMBERS in those boxes, nothing else, otherwise the calculations will not work!!

Compilation of different smaller workbook data, entered at the intermediary level into the country workbook can be done by careful copying and pasting. It is always best done by using “PASTE SPECIAL” from the EDIT menu, not a simple PASTE, to avoid disturbing formats and formulas.

- after entering the name of the country, designation of the administrative divisions and year, first copy the set-up sheets with names of lowest administrative division, names of labs.. .to the correct columns (two columns to the left of the controller names column). Then add the name(s) of the second level administrative division in the column to the left of the lowest division names, and copy downwards as far as there are lab names for this administrative division. Do the same for the names of the highest level divisions in the left-most column. Finally, copy the names of the controllers in the right-hand column. Do the same for each of the smaller workbooks comprising an administrative division, each time starting on the first empty line

- after copying the names to the set-up sheet, they will now appear in all data sheets. Now copy the data to these sheets workbook by workbook and quarter by quarter, making sure to select only the area containing numbers and controller names (all at once, comprising peripheral lab data, controller names and controller data from left to right and all labs from top to bottom. Clicking PASTE SPECIAL after selecting, copying and placing the cursor in the left hand uppermost destination cell will bring up a sub-menu, from where “Values” must be chosen. Make sure the paste special is done in the correct place, just to the right of the name of the first lab, checking after the paste that the correct area is covered (grey). Do the same sheet after sheet for the Q1 to Q4 and Q1P to Q4P. And repeat this for workbook after workbook, making sure to paste in the correct sheet and on the correct line.

How to prepare reports and listings

All sheets can be printed after selecting the printing area and setting the printing formats, as usual in EXCEL. Often the empty lines at the bottom will first need to be hidden, by selecting the complete line under the last laboratory, extending the selection downwards to just above the line with column totals, and then clicking the right mouse tab choosing “hide”. In the same way a print-out can be made for just one of the administrative units if needed, provided these were entered together in the set-up sheet or filtered in the analysis sheet. If not, an intermediary copying step will be necessary since sorting directly in these sheets will not work:

- select the whole sheet by clicking in the left hand corner where the row and column numbers meet, and “copy”

- open a new sheet, click in the same corner tab and choose “PASTE SPECIAL”. First select “Values”, then without undoing the selection, select again “Formats”.

- this produces a copy with values only without formulas, in which you can freely sort by selecting the necessary lines, as usual in Excel.

The “Analysis_Labs” sheet has filter tabs that allows for several types of selections and combinations of selections. For instance, here it is possible to select one administrative unit or any level, with all its sub-divisions and laboratories, which is not possible in the automatically generated sub-division reports that can show only one administrative level. However, it is not possible to sort. One can also use a selection for level of performance, for instance to obtain a list of labs with less than 0.75 detection relative to the controllers. Several selections can be combined. These selections can then be printed or copied to another location, for further modifications, i.e. alphabetical sorts. To use these filter functions:

- to select for a certain value: click on the arrow in the column heading on which you want to select, i.e. names of the districts. Look down the alphabetically sorted list, and click the name you want. Now only the lines of this district are shown

- another option is to use filter criteria, i.e. to see only the labs responding to certain performance criteria. Click the arrow, but choose “Custom”. This brings up a selection box, where the selection values have to be defined. Combinations can be used.

- the active filter tabs are visible since their arrows become blue. The selections can be undone by clicking again on their arrow and selecting “All”.

Some sheets, for example the Annual Performance sheet, have conditional formats for some columns that will automatically tag by a color the labs remaining under a pre-defined level of performance, or with values out of the normal range. These formats can be changed by the user without undoing the protection. Select the range of cells or column, then choose “Conditional formatting” from the Format menu, and enter the criteria and tagging colors you want.

How to prepare a fresh workbook for the coming year

This is best done from the existing book of the preceding year, saved as a new file name, so that the baseline data do not need to be entered again (leaving the set-up as it is, except for a change of the year). It will then be sufficient to select the data entered manually in sheets Q1 to Q4 and Q1P to Q4P, deleting them.  Be sure to save the workbook for the new year under a different name. One selection per sheet is possible since data cells are not separated by any protected formulas. Should an error message appear, it means that the selection was extended too far, for instance including the names of the laboratories.