On-site supervision short checklist exercise - TASK

Please refer to the short checklist below and analyze information which was obtained during the on-site supervision visit to Laboratory A of the District D. Please count the laboratory workload and calculate the percentage of positive suspects smears and the percentage of positive follow-up smears; provide your overall remarks and recommend corrective actions if needed.

On-site supervision short checklist

Laboratory:							 	LABORATORY A

District/Administrative Unit:					District D

Number of Microscopists/Technicians:		one laboratory technician / microscopist

Qualifications of current staff: 				basic training for technicians in 2000

Supervisor/Head of Laboratory: 				Dr. H.L.

Date of Visit: 								November 11, 2006

Visiting Supervisor:						Dr. V.S.

Item				Adequate/		Problems Identified

Acceptable

SOP		Y	N		SOPs are disorganized, and are not kept at the place of use

Separate area for TB work		 Y	N		__________________________________________________

Separate tables for specimen 		Y	N		__________________________________________________

receipt/smear preparation/microscopy

General order/ cleanliness		Y	N		cleanliness is poor, lab is disorderly and dirty

Power supply		Y	N		power often not available during working hours

Running water supply		Y	N		__________________________________________________

Waste containers with lid		Y	N		waste containers are without lids

Waste disposal by

autoclave/burning/buried		Y	N		waste is thrown out in the corner of the yard

Balance/		Y	N  		not applicable, stains are received from intermediate lab

measuring glassware/ pure water

(if stains are prepared)

Adequate Stock & Supply of:

Specimen containers 		Y	N		__________________________________________________

Slides					Y	N		__________________________________________________

Stains					Y	N		__________________________________________________

Slide boxes				Y	N		__________________________________________________

Microscopes					Y	N		an old monocular microscope with mirror is available

Laboratory register				Y	N		 positive results are not written in red

Laboratory forms				Y	N		__________________________________________________

Overall quality of smears		Y	N		many smears are too thick

Timely reporting of results 		Y	N		__________________________________________________

to clinicians

Personnel					Y	N		no NTP training in AFB smear microscopy

Is QC using positive and negative control slides performed as required by the NTP?		√Yes     No

____________________________________________________________________________________

Are all slides kept as required by the NTP EQA Program?					  Yes    √ No

all slides are kept but some of them have positive result written on them

Are slides properly stored in slide boxes?					  √Yes     No

____________________________________________________________________________________

Workload (last 3 months)

| Number of smears: |  | Number of suspects smears: |  | Number of follow up smears: |  |
| --- | --- | --- | --- | --- | --- |
| Total: 736 |  | Total: 655 |  | Total: 81 |  |
| # Pos 61 | # Neg 675 | # Pos 61 (incl. only few low positives) | # Neg 594 | # Pos 0 | # Neg 81 |

Average number of smears read by each technician per day?

Proportion of positive (including scanty) suspects smears among all suspects smears examined?

Proportion of positive (including scanty) follow-up smears among all follow-up smears examined?

Rechecking and/or Panel Testing Results (refer to feedback form)

Have any performance problems (based on criteria set by NTP) been identified through rechecking or panel testing?  											  Yes  √  No

No errors were found by rechecking during the whole year.

Has corrective action been adequately implemented?		  Yes     No

__________________________________________________________________________________Overall Remarks:

____________________________________________________________________________________________________________________________________________________________________________

Action required:

______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________